DIR – Trip Equipment SOP v4.0

(ARW 1-2-2007)

- Backplate

  - Check arm straps  - enough room to fit thumbs under easily with drysuit on?

  - D-Ring location – comfortable? Check for muscle memory

  - If not diving argon, remove argon straps

  - Check back pocket stays – are they tight?

  - Check length on butt strap

  - Check crotch d-ring

  - Make sure light “weight belt bucket” is on for light

- Explorer Wing

  - Check back dump valve

    - Inflate until overpressure valve releases

    - Clean of debris

    - Check mating surfaces and spring tension.

  - Check inflator valve (connect to air)

  - Check deflator valve (connect to air)

  - Check for leaks –

    - inflate fully and listen for leaks

    - Leave full inflated over night.

- Drysuit

  - Pressure test suit with cups

  - Check inflation valve

  - Rebuild/Check seats on dump valve

  - Visually inspect seals (neck and wrist for fit)

  - air-test –pvalve

  - Wax zipper

  - Test ease of zipper

  - Test fit with rig on

  - Quick double-check – do I have either an argon bottle set up or the appropriate inflation hose on my regs?

- Weight Belt

  - Check buckle

  - Check length

- Fins

  - Check spring heel springs - bent too much out of shape?

  - Check Spring Heel bolt connections (on tight? Bent?)

- Depth Gauge/Computer

  - Check battery

  - Check backlighting

  - Check integrity of bungee

  - Check accurate time

- Compass

  - Check integrity of bungee

  - Compare to second Compass to verify accuracy

- Ironman Watch

  - Check battery

  - Check backlighting

  - Check accurate time

- Xnotes

  - Check for basic deco profiles @ appropriate depth

  - Check pencil for an end

- Jump/Deco Spool

  - Check knots

  - Run length and check integrity of line

- Reel

  - Check knots

  - Verify that there’s a double-ender on there

  - Run length and check integrity of line

- Knife

  - Snug inside sheath?

  - Check sharpenss of blade

- Apeks DS4 Regulator

  - IP Tests

    - Check IP is set correctly (120 psi)

    - Under FULL purge check “droop” (less that 10 PSI or so)

    - Release purge and check for “IP Creep”

  - Check tightness of port plugs

  - Check connections of hoses

  - Check number of hoses on each reg for DIR rigging

  - Remember to add inflation if not diving argon (no helium on B.G.)

  - Remove and check the integrity of o-ring

  - Pressure Gauge

    - Check pressure gauge: run through 300 pounds of air

    - Check ease of bolt snap slide on pressure gauge

    - Check integrity of bolt-snap attachment

- Apeks ATX50 2nd Stage

  - Remove cover, replace just over finger tight

  - Clean of sand and debris

  - Check valve seats

  - Check integrity of purge-arm

  - Check integrity of bolt snap attachment

  - Check ease of bolt-snap slide

- Apeks TX40 2nd Stage

  - Remove cover, replace just over finger tight

  - Clean of sand and debris

  - Check valve seats

  - Check integrity of purge-arm

- Mask

  - Check lens retainer integrity

  - Check strap for wear

- Backup Mask

  - Same

- Helios 9 Dive light

  - Adjust Goodman Handle to be the right size with/without gloves as needed

  - Burn test light

  - Check bulb for discoloration

  - Check that bulb is seated

  - Check seals on test tube

  - Check bolt-snap connection

  - Check ease of bolt-snap slide

  - Check solder points on battery attachment

- Halcyon Charger

  - Charge all batteries before trip – if Lithium, charge to full, disconnect for 30 seconds, re-charge

- Backup Light

  - Replace batteries with new set

  - Clean gunk off o-rings and mating areas & reapply non-petroleum grease

  - Check O-ring for nicks, etc

  - Make sure bulb works.

  - Inspect for any signs of rust/corrosion.

- Tanks

  - Check for current visual / hydro! (Opps!)

  - Check that valve holds gas when OPEN with reg attached.

  - Check knobs to see if handwheel nut is almost falling off!

  - Check valve for signs of being dropped. If so, make sure reg still seats properly

  - Attach regs, isolate, run down right post 100psi, open isolator to see if gas moves properly from one side to the other

  - Re-analyze & Mark gas (what was I diving last?)

- 40cuft O2 – Same as all tanks

- Reg - O2 – Same as all the other regs!

- 2nd Stage - O2  - Same as all the other regs!